NELSON MUFFLER #s             LIST FOR N.A. MOTORS  (X OVER LISTS FOUND ONLINE)
(TYPE 1 = END IN ON C / OTHER END OUT ON C)  (TYPE 2 = END IN OFFSET / OTHER END IN OFFSET)(TYPE 3 = END IN / SAME END OUT) (TYPE 4 END IN ON C / OTHER END OUT OFFSET)
(NOISE IN DECIBELS @ 50 FEET)(DIMENSIONS IN INCHES)(RESTRICTION IN “of HG @ ENGINE )
	PART #
	STYLE
	OUTLETS
	BODY SIZE
	O.A.L.
	NOISE
	RESTRICTION

	6-71 NA
	SINGLE MUFFLER
	
	
	
	
	

	86115M
	3
	4.0 – 4.0
	10X15 OVAL
	26
	79
	3.7

	
	
	
	
	
	
	

	6V-71 NA
	SINGLE MUFFLER
	
	
	
	
	

	86125M
	2
	4.0 – 4.0
	10X15 OVAL
	26
	83
	1.8

	86543M
	2
	4.0 – 5.0
	10X15 OVAL
	27
	82
	2.3

	
	
	
	
	
	
	

	6V-71 NA
	DUAL MUFFLERS
	
	
	
	
	

	86159M
	2
	3.0-3.0
	8.3X11.5 OVAL
	26
	81
	3.0

	86512M
	2
	3.50-4.0
	8.3X11.5 OVAL
	28
	82
	1.2

	86106M
	2
	3.50 – 3.50
	8.3X11.5 OVAL
	26
	83
	1.9

	
	
	
	
	
	
	

	8V-71 NA
	DUAL MUFFLERS
	
	
	
	
	

	86512M
	2
	3.5 – 4.0
	8.3X11.5 OVAL
	
	83
	1.8


Other mufflers (to fit) but NOT rated for DD 2 STROKE

	86560M
	1
	4.0 – 4.0
	8.9 RND
	20

	D13-0123
	1
	5.0 – 5.0
	8.9 RND
	23

	86121M
	3
	5.0 – 5.0
	10X15 OVAL
	26

	
	
	
	
	

	86144M *
	1
	4.0 – 4.0
	9.0 RND
	11.5

	86145M *
	1
	5.0 – 5.0
	9.0 RND
	11.5

	
	
	
	
	


* = RESINATORS
